AUX-9

Autonomous Hybrid Intelligence Vehicle System

PHASE 1 — SYSTEM ARCHITECTURE DESIGN DOCUMENT

Classification: Engineering Design  |  Version: 1.0  |  Status: Draft

| Document Info | Details |
| --- | --- |
| Project | AUX-9 Autonomous Vehicle |
| Phase | Phase 1 — Architecture Design |
| Scope | System Architecture Only |
| Core Boards | Raspberry Pi + STM32F411 + ESP32 |
| Budget (extra) | INR 7,000 – 8,000 |
| Next Phase | Phase 2 — Detailed HW Design & BOM |

# 1.  Project Vision

## 1.1  What is AUX-9?

AUX-9 is a low-cost, distributed autonomous hybrid intelligent vehicle designed to demonstrate real-world autonomous navigation, obstacle avoidance, and WiFi-based manual control using a tightly integrated multi-board architecture. The system is engineered to be fully buildable within a constrained budget while maintaining professional-grade design decisions, safety mechanisms, and architectural clarity.

AUX-9 is not a toy or a simple RC car. It is a proof-of-concept for distributed embedded intelligence applied to autonomous mobility — combining computer vision, sensor fusion, real-time embedded control, and wireless communication into a single coherent vehicle system.

## 1.2  Problem Statement

Most low-cost autonomous vehicle projects attempt to run all computation — AI inference, motor control, and communication — on a single microcontroller or single-board computer. This creates three fundamental engineering problems:

- Single-board overload: AI workloads (camera processing, path planning) consume all CPU resources, making real-time motor control unreliable or impossible.

- Safety compromise: Without dedicated safety hardware, a crashed AI process has no independent layer to halt the motors. The vehicle can become dangerous.

- Scalability bottleneck: A monolithic architecture cannot be upgraded incrementally. Improving AI requires replacing the entire control system.

AUX-9 solves these problems by distributing intelligence across three specialized boards, each optimized for its domain of responsibility.

## 1.3  Why This Architecture?

The three-board distributed architecture is chosen because it mirrors industry practice in embedded systems design: separation of concerns, hard real-time isolation, and fault containment. Each board operates within its competency boundary, communicates over simple serial protocols, and can fail independently without collapsing the entire system.

This architecture also allows parallel software development across all three boards, incremental hardware upgrades, and clear fault attribution during debugging.

## 1.4  Why Raspberry Pi + STM32F411 + ESP32?

Rather than relying on a single board, AUX-9 deliberately assigns work to the board best suited for that class of task:

- High-level AI processing, computer vision via OpenCV, sensor fusion in Python, and mission management. The Pi offers Linux, USB, camera interface, and enough CPU/GPU headroom for AI inference. It is not suitable for hard real-time control due to OS scheduling jitter.Raspberry Pi 4:

- Hard real-time motor control, safety watchdog, and sensor polling. It runs bare-metal embedded C with deterministic interrupt timing. No OS jitter, no process scheduling, no crashes. It is the final authority over all actuators.STM32F411 (Cortex-M4 @ 100 MHz):

- WiFi communication, web dashboard hosting, and manual control relay. Its built-in WiFi radio and async web server capability make it the ideal communication gateway. It bridges the human operator and the vehicle with minimal latency.ESP32:

Used together, they create a system that is individually simple on each board but collectively powerful and resilient.

# 2.  System-Level Architecture

## 2.1  Subsystem Overview

AUX-9 is divided into six primary subsystems. Each subsystem has a clear owner, defined inputs and outputs, and a focused set of responsibilities.

| Subsystem | Owner Board | Inputs | Outputs | Key Function |
| --- | --- | --- | --- | --- |
| Perception | Raspberry Pi | Camera, US, ToF, IMU, GPS, IR | Obstacle map, pose estimate, environment model | Sense the world |
| Decision | Raspberry Pi | Perception outputs, mode state | Motion commands to STM32 | Plan and decide |
| Control | STM32F411 | Commands from Pi/ESP32 | PWM to L298N, motor torque | Execute motion safely |
| Communication | ESP32 | WiFi from user, telemetry from STM32 | Commands to STM32, status to user | Bridge user and vehicle |
| Safety | STM32 (primary) | Watchdog, sensor faults, comm timeouts | Emergency stop, fault flags | Prevent harm at all times |
| Power | Shared (passive) | Battery voltage | Regulated rails to all boards | Stable power distribution |

## 2.2  Perception Subsystem

### Purpose

The perception subsystem is responsible for gathering raw information about the physical environment and converting it into structured data that the decision engine can act upon. It is the sensory layer of AUX-9.

### Inputs

- Pi Camera: raw RGB video frames at 15–30 fps

- MPU6050: 6-DOF inertial measurements (acceleration + gyroscope)

- VL53L0X: precise laser-based range measurements up to 2 meters

- NEO-6M GPS: NMEA sentences providing latitude, longitude, altitude, and fix quality

- HC-SR04 (x4): acoustic echo-based distance measurements in four directions

- IR sensors (x2): binary obstacle or edge detection for backup coverage

### Outputs

- Obstacle proximity map (directional distance to nearest obstacle per sector)

- Current heading and angular velocity from IMU

- Precise short-range distance to front obstacle from ToF

- Global position estimate (when GPS fix available)

- Detected object classes and bounding boxes from camera pipeline

### Responsibilities

- Capture and preprocess sensor data at required rates

- Validate sensor readings against plausible ranges

- Flag stale or anomalous data to the fusion engine

- Timestamp all sensor readings for synchronization

## 2.3  Decision-Making Subsystem

### Purpose

The decision subsystem fuses perception outputs into a unified world model and computes the next best action for the vehicle. It is the brain of AUX-9.

### Inputs

- Obstacle map and sensor data from perception layer

- Current operating mode (autonomous, manual, degraded)

- Mission waypoints or target heading (if applicable)

### Outputs

- Motion commands: speed (0–100%), direction (forward/reverse/left/right/stop)

- Mode transition requests (e.g., escalate to emergency stop)

- Status and telemetry data

### Responsibilities

- Perform sensor fusion across all perception inputs

- Run obstacle avoidance algorithm (reactive avoidance in Phase 1)

- Determine safe speed and heading

- Issue formatted UART command packets to STM32

## 2.4  Control Subsystem

### Purpose

The control subsystem translates abstract motion commands into precise electrical signals to the motors. It operates in hard real-time and has no tolerance for latency or jitter.

### Inputs

- Motion command packets from Raspberry Pi (UART)

- Manual command packets from ESP32 (UART)

- Sensor readings from HC-SR04 and IR (direct to STM32)

### Outputs

- PWM signals to L298N motor driver

- Direction control signals (IN1–IN4) to L298N

- Telemetry packets to ESP32 (speed, mode, fault flags)

### Responsibilities

- Decode and validate incoming command packets

- Generate smooth PWM transitions (acceleration ramping)

- Enforce minimum safe distances using local ultrasonic data

- Execute emergency stop with highest interrupt priority

- Report vehicle state to ESP32 at 10 Hz

## 2.5  Communication Subsystem

### Purpose

The communication subsystem provides the wireless link between the human operator and the vehicle, and relays manual commands and telemetry between the web interface and the control hardware.

### Inputs

- HTTP / WebSocket commands from operator's browser

- Telemetry packets from STM32 via UART

### Outputs

- Formatted UART command packets to STM32

- Real-time telemetry dashboard rendered in browser

### Responsibilities

- Host a WiFi access point and web server

- Translate web UI inputs into UART-compatible command packets

- Forward vehicle telemetry to the operator's dashboard

- Relay mode-switch commands (AUTO/MANUAL/E-STOP) to STM32

## 2.6  Safety Subsystem

### Purpose

The safety subsystem is the final line of defense against vehicle damage or danger. The STM32 is the primary safety authority — no AI decision or network command can override a hardware-level safety trigger.

### Inputs

- Ultrasonic sensor distance readings (direct on STM32)

- IR sensor obstacle flags (direct on STM32)

- Watchdog timer expiry

- Communication timeout counter

- Battery voltage monitor

### Outputs

- Emergency stop command to L298N (immediate motor disable)

- Fault flag packet to Raspberry Pi and ESP32

### Responsibilities

- Monitor all critical thresholds continuously

- Trigger E-STOP independently of Pi or ESP32

- Enforce communication timeout (vehicle stops if Pi goes silent)

- Report fault codes for diagnosis

## 2.7  Power Subsystem

### Purpose

The power subsystem provides stable, isolated power to each domain of the vehicle, preventing motor noise and voltage drops from affecting sensitive electronics.

### Responsibilities

- Supply regulated 5V to Raspberry Pi, ESP32, and sensors

- Supply regulated 3.3V where required (ESP32 GPIOs, STM32 peripherals)

- Supply unregulated battery voltage to L298N motor driver

- Filter motor-switching noise using bulk capacitors

- Monitor battery voltage for low-battery safety response

# 3.  Distributed Board Responsibility Mapping

| Board | Primary Role | Real-time Class | Key Responsibilities |
| --- | --- | --- | --- |
| Raspberry Pi 4 | AI & Decision Engine | Non / Soft real-time | Camera pipeline, sensor fusion, path planning, mission manager, mode arbiter |
| STM32F411 | Real-time Control & Safety | Hard real-time | PWM motor control, safety watchdog, command parsing, emergency stop authority |
| ESP32 | Communication & Manual I/O | Soft real-time | WiFi access point, web dashboard, manual command relay, telemetry display |

## 3.1  Raspberry Pi — AI and Decision Engine

The Raspberry Pi is the highest-level processor in AUX-9. It is responsible for all tasks that require significant computation, data integration, or AI inference. These tasks are classified as non-real-time or soft real-time — meaning they can tolerate some scheduling jitter as long as average throughput requirements are met.

- Camera capture and preprocessing (OpenCV, 15–30 fps)

- Object detection using TFLite or classical CV techniques

- IMU and GPS data acquisition via I2C and UART

- VL53L0X ToF range reading via I2C

- Sensor fusion: merging all perception data into an environment model

- Path planning and obstacle avoidance decision logic

- Mode arbitration: selecting IDLE/AUTO/DEGRADED modes

- Sending formatted motion commands to STM32 via UART

- Receiving telemetry from STM32 for logging and monitoring

The Pi does NOT directly control motors. It issues intent (e.g., 'move forward at 70%') and the STM32 executes.

## 3.2  STM32F411 — Real-Time Control and Safety

The STM32F411 is the lowest-level controller and the only board with direct authority over the motors. All tasks on the STM32 are hard real-time — they must execute within fixed, deterministic timing windows or the vehicle is unsafe.

- PWM generation to L298N motor driver (via TIM peripheral)

- Direction control via GPIO (IN1–IN4 of L298N)

- HC-SR04 trigger and echo measurement (via timer capture)

- IR sensor polling

- UART packet reception from Raspberry Pi and ESP32

- UART telemetry transmission to ESP32

- Safety watchdog: independent hardware timer

- Emergency stop execution: highest-priority interrupt

- Communication timeout monitoring

The STM32 does NOT run an operating system. All logic is interrupt-driven or in a deterministic superloop. This ensures microsecond-level response to safety events.

## 3.3  ESP32 — Communication and Manual Interface

The ESP32 bridges the human operator and the vehicle. Its tasks are soft real-time — there is some latency tolerance, but commands must arrive fast enough to feel responsive (target: < 100 ms from input to motor response for manual control).

- WiFi access point hosting (IEEE 802.11 b/g/n)

- Async web server with control dashboard (HTML/JS)

- WebSocket or HTTP polling for real-time telemetry display

- Accepting joystick/button inputs from operator's browser

- Translating UI inputs to UART command packets for STM32

- Relaying mode switch commands (AUTO/MANUAL/E-STOP)

- Forwarding STM32 telemetry to the web dashboard

The ESP32 does NOT perform AI or sensor fusion. It is a communication relay with a web-facing interface.

# 4.  Sensor Role Mapping

| Sensor | Board | Data Provided | Read Rate | Subsystem | Failure Mode | Failure Response |
| --- | --- | --- | --- | --- | --- | --- |
| HC-SR04 (x4) | STM32 | Distance 2–400 cm | 20 Hz | Perception / Safety | No pulse / frozen value | Mark zone as 'unknown-danger', halt if front sensor fails |
| MPU6050 | Raspberry Pi | Accel + Gyro (6-DOF) | 50–100 Hz | Perception / Fusion | I2C timeout / garbage data | Disable IMU fusion channel, degrade to dead-reckoning or halt |
| Pi Camera | Raspberry Pi | RGB frames (480p+) | 15–30 fps | Perception (vision) | No frames / crash | Drop to sensor-only nav or enter degraded mode |
| VL53L0X | Raspberry Pi | Precise range 0–2 m | 30–50 Hz | Perception / Safety | I2C timeout | Rely on ultrasonic for short-range; log fault |
| NEO-6M GPS | Raspberry Pi | Lat/Lon/Alt, NMEA | 1–5 Hz | Decision (position) | No fix / weak signal | Indoor: GPS optional. Outdoor: enter wait-fix state before moving |
| IR Sensors (x2) | STM32 | Binary obstacle / edge detect | 20 Hz | Safety / Backup | Always-high or always-low | Flag sensor faulty; secondary backup so degrade gracefully |

## 4.1  HC-SR04 Ultrasonic Sensors (x4)

- Primary obstacle detection. Provides 360-degree coverage when deployed at front, rear, left, and right of chassis.Why needed:

- STM32F411 (timer input capture for echo measurement)Connected to:

- Distance in cm (range 2–400 cm, practical reliable range 2–200 cm)Data provided:

- Each sensor triggered at 20 Hz (staggered to prevent crosstalk)Read rate:

- Perception and Safety — STM32 uses readings both for telemetry to Pi and for local safety enforcementSubsystem:

- If front sensor fails, STM32 marks front zone as 'unknown danger' and prohibits forward motion until sensor is confirmed operational or operator manually overrides.Failure response:

## 4.2  MPU6050 IMU

- Provides orientation (pitch, roll, yaw rate) and acceleration for sensor fusion. Essential for detecting unintended rotation, ramp conditions, or collision impact.Why needed:

- Raspberry Pi (I2C bus, address 0x68)Connected to:

- 3-axis acceleration (m/s²) + 3-axis gyroscope (°/s) at 50–100 HzData provided:

- Perception and Sensor FusionSubsystem:

- IMU fusion channel disabled; system degrades to camera + ultrasonic only; speed reduced as precaution.Failure response:

## 4.3  Raspberry Pi Camera

- Enables visual perception — object detection, path following, scene understanding. Computer vision is the most context-rich sensory input available at this price point.Why needed:

- Raspberry Pi (CSI camera interface)Connected to:

- RGB frames, 480p or higher, at 15–30 fpsData provided:

- Perception (vision pipeline)Subsystem:

- Vision pipeline disabled; system drops to sensor-only navigation in degraded mode; speed significantly reduced; operator warned.Failure response:

## 4.4  VL53L0X Time-of-Flight Sensor

- Provides high-accuracy short-range distance (0–2 m) using laser ranging. Fills the accuracy gap that ultrasonic sensors have at very short distances and narrow beam angles.Why needed:

- Raspberry Pi (I2C bus)Connected to:

- Distance in mm, up to 2 m, at 30–50 HzData provided:

- Perception and Safety (close-range collision prevention)Subsystem:

- Fall back to ultrasonic for close-range detection. Log fault. No immediate stop unless ultrasonic also fails.Failure response:

## 4.5  NEO-6M GPS

- Provides absolute global positioning for outdoor navigation and geo-referenced mission waypoints.Why needed:

- Raspberry Pi (UART, NMEA sentences at 9600 baud)Connected to:

- Latitude, longitude, altitude, HDOP, satellite count, fix type at 1–5 HzData provided:

- Decision-making (position awareness for path planning)Subsystem:

- For indoor use, GPS is optional and system continues normally. For outdoor waypoint missions, vehicle enters a wait-for-fix state and does not begin navigation until fix quality is confirmed.Failure response:

## 4.6  IR Sensors (x2)

- Backup obstacle and edge detection. Digital binary output requires no computation. Provides a last-resort hardware-level obstacle flag independent of the main sensor pipeline.Why needed:

- STM32F411 (GPIO digital input)Connected to:

- Binary: obstacle present / not present, updated at 20 HzData provided:

- Safety (backup)Subsystem:

- Stuck-high: treated as permanent obstacle in that zone (conservative). Stuck-low: flagged as faulty and excluded. Because IR is a backup sensor, a single IR failure does not trigger E-STOP.Failure response:

# 5.  Actuator and Control Architecture

## 5.1  Motor Control Architecture

AUX-9 uses a differential drive system with two independently controlled DC motors. This is the simplest and most reliable drive architecture for a ground vehicle at this cost level. Steering is achieved by varying the relative speed and direction of the left and right motors — no separate servo is required.

## 5.2  L298N Motor Driver

The L298N is a dual full H-bridge motor driver capable of driving two DC motors independently. It accepts PWM signals from the STM32 for speed control and digital direction signals for forward/reverse. Key characteristics:

- Operating voltage: 5–35V motor supply, 5V logic

- Maximum current: 2A per channel, 3A peak

- PWM frequency: 1–25 kHz (STM32 configured at ~10 kHz for clean operation)

- Direction: controlled via IN1/IN2 (motor A) and IN3/IN4 (motor B)

Note: The L298N has approximately a 2V voltage drop per H-bridge due to its bipolar transistor design. At a 7.4V battery input, motors receive approximately 5.4V. This must be accounted for in motor selection.

## 5.3  Command Flow from AI Decision to Physical Motion

Raspberry Pi: decision engine computes desired speed and direction

→ Pi formats UART packet: [START][CMD_TYPE][SPEED][DIRECTION][CHECKSUM][END]

→ Pi transmits packet to STM32 via UART at ~100 Hz command rate

STM32: command parser receives packet, validates checksum

→ Safety monitor checks: distance OK? watchdog OK? battery OK?

→ If all safe: motor control task updates PWM duty cycle

→ TIM peripheral generates PWM on ENA/ENB pins of L298N

→ GPIO sets IN1-IN4 for direction

→ L298N drives motor windings accordingly

→ Motors produce torque, vehicle moves

## 5.4  Feedback and Fail-Safe Flow

AUX-9 Phase 1 does not use wheel encoders (cost constraint). Feedback is derived from IMU (heading drift) and ultrasonic sensor distance changes. The fail-safe flow is:

- If Pi stops sending commands for > 500 ms → STM32 watchdog timeout → immediate motor stop

- If ultrasonic detects obstacle < 20 cm (configurable threshold) → STM32 overrides all commands → E-STOP

- If battery ADC reading falls below threshold → STM32 sets low-battery flag, reduces max speed to 30%, notifies Pi

- If STM32 asserts E-STOP, it sends fault packet to ESP32 for operator notification

# 6.  Data Flow Architecture

## 6.1  Autonomous Mode Data Flow

┌─────────────────────────────────────────────────────────────────────┐

│                     AUTONOMOUS MODE DATA FLOW                       │

└─────────────────────────────────────────────────────────────────────┘

[Pi Camera]──────────────────────────────────────────────────────────┐

[MPU6050  ] (I2C)──────────────────┐                                 │

[VL53L0X  ] (I2C)──────────────────┤                                 │

[NEO-6M GPS] (UART)────────────────┤                                 │

▼                                 ▼

┌────────────────────┐    ┌──────────────────────┐

│  Sensor Fusion     │    │  Camera Pipeline     │

│  (Raspberry Pi)    │    │  Object Detection    │

└────────┬───────────┘    └──────────┬───────────┘

│                           │

└───────────────────────────┘

│

▼

┌──────────────────────────┐

│   Decision Engine        │

│   (Obstacle Avoidance,   │

│    Path Planning)        │

└─────────────┬────────────┘

│ UART Command Packet

▼

┌──────────────────────────┐

│   STM32F411              │

│   (Command Parser +      │

│    Safety Monitor)       │

└──────┬───────────────────┘

│ PWM + Direction

▼

┌──────────────┐

│    L298N     │

└──────┬───────┘

│

┌───────────┴────────────┐

▼                        ▼

[Left Motor]            [Right Motor]

[HC-SR04 x4]──── direct to STM32 (safety enforcement layer)

[IR Sensors ]──── direct to STM32 (safety enforcement layer)

## 6.2  Manual Mode Data Flow

┌─────────────────────────────────────────────────────────────────────┐

│                       MANUAL MODE DATA FLOW                         │

└─────────────────────────────────────────────────────────────────────┘

[Operator Browser]

│ HTTP / WebSocket

▼

┌─────────────────┐

│     ESP32       │

│  (Web Server /  │

│  Command Parser)│

└────────┬────────┘

│ UART Command Packet

▼

┌─────────────────────────────┐

│         STM32F411           │

│ (Command Parser + Safety    │

│  Monitor — safety ALWAYS on)│

└──────────────┬──────────────┘

│ PWM + Direction

▼

┌─────────────┐

│    L298N    │

└──────┬──────┘

┌───────────┴────────────┐

▼                        ▼

[Left Motor]          [Right Motor]

[HC-SR04 x4]──── still active in manual mode (safety cannot be disabled)

[IR Sensors ]──── still active in manual mode

## 6.3  Telemetry and Feedback Flow

┌─────────────────────────────────────────────────────────────────────┐

│                    TELEMETRY & FEEDBACK FLOW                        │

└─────────────────────────────────────────────────────────────────────┘

[HC-SR04 distances]

[IR sensor states  ]────────▶ STM32 Telemetry Reporter

[Mode / fault flags]                   │

[Battery ADC reading]                  │ UART Telemetry Packet (10 Hz)

▼

┌─────────────────────┐

│     ESP32           │

│  (Telemetry Sender) │

└──────────┬──────────┘

│ WebSocket / HTTP

▼

[Operator Dashboard]

(live distance, speed,

mode, fault status)

STM32 ──────────────────────▶ Raspberry Pi (UART)

(ACK, fault codes, sensor readings for fusion loop)

# 7.  Mode Architecture

| Mode | Entry Condition | Behaviour | Exit Condition | Controlling Board |
| --- | --- | --- | --- | --- |
| INIT | Power on | All boards self-test, sensor checks, UART handshake | All checks pass → IDLE | All boards jointly |
| IDLE | INIT success; manual stop; end of mission | Motors off, WiFi active, telemetry streaming | User command or mission loaded | Raspberry Pi |
| AUTONOMOUS | User selects AUTO via web UI or mission upload | Pi drives, STM32 executes, full sensor fusion active | Goal reached; E-STOP; fault; manual override | Raspberry Pi (commands) / STM32 (execution) |
| MANUAL | User sends manual override via web UI | ESP32 relays joystick commands directly to STM32; Pi monitors only | User releases control; timeout; E-STOP | ESP32 → STM32 |
| EMERGENCY STOP | Obstacle < threshold; watchdog timeout; low battery; command | Motors immediately zeroed; brakes; Pi notified; telemetry emergency flag | Only manual clear from web UI or hardware reset | STM32 (final authority) |
| DEGRADED | One or more non-critical sensor failures | Reduced speed, limited autonomy, operator warned | Sensor restored; manual intervention; or fault escalates to E-STOP | Raspberry Pi (monitors) / STM32 (executes) |

## 7.1  Mode Transition Diagram (Text)

[POWER ON]

│

▼

┌─────────┐   All checks pass

│  INIT   │──────────────────────▶ IDLE

└────┬────┘

│ Any critical check fails

▼

┌─────────┐

│  FAULT  │ (E-STOP immediately)

└─────────┘

IDLE ──────────────────────────────────────────────────┐

│  User selects AUTO                                  │

│                                                     │

▼                                                     ▼

AUTONOMOUS ─── obstacle / fault / E-STOP ──▶  EMERGENCY STOP

│  Manual override command                            ▲

▼                                                     │

MANUAL ─────────────────── obstacle / fault ────────────┘

│  Release control / timeout

▼

IDLE

AUTONOMOUS or MANUAL ─── sensor fault ──▶  DEGRADED

DEGRADED ──────────────── sensor restored ──▶  back to prev mode

DEGRADED ──────────────── critical fault ──▶  EMERGENCY STOP

EMERGENCY STOP ─── only via manual web UI clear or hardware reset

# 8.  Safety Architecture

## 8.1  Safety Hierarchy

The safety architecture of AUX-9 follows a strict priority hierarchy. At no point can a higher-level component (Pi, ESP32, or operator) override a lower-level safety mechanism. The STM32 is the final hardware authority over all motor outputs.

Priority (1 = highest):

1. STM32 hardware watchdog (IWDG)    — cannot be disabled by software

2. STM32 obstacle threshold check     — immediate motor cut

3. STM32 communication timeout        — motors stop if Pi or ESP32 silent

4. STM32 battery undervoltage         — reduces speed, then stops

5. Raspberry Pi safety checks         — sensor fusion anomaly detection

6. ESP32 E-STOP button                — operator manual intervention

7. Raspberry Pi mission abort logic   — goal-level safety decisions

## 8.2  Emergency Stop Logic

Emergency stop is the highest-priority action in the system. When triggered, it is instantaneous and cannot be pre-empted by any software process.

- Trigger sources: ultrasonic distance < threshold, IR obstacle detected, watchdog timeout, low battery, UART E-STOP command from ESP32 or Pi

- Response: STM32 immediately sets PWM to 0% on both motor channels, sets both direction pins to LOW (brake state on L298N)

- Notification: STM32 transmits E-STOP fault packet to both Pi and ESP32

- Recovery: Only possible via explicit CLEAR command from operator via web UI, followed by confirmation that obstacle is resolved

## 8.3  Communication Timeout

The STM32 maintains a watchdog counter for each UART source (Pi and ESP32). If no valid packet is received within a configurable window (default: 500 ms), the corresponding source is flagged as silent and:

- If Pi is silent in AUTO mode: motors stop, mode transitions to IDLE

- If ESP32 is silent in MANUAL mode: motors stop, mode transitions to IDLE

- If both are silent: motors stop, fault flag set, vehicle waits

## 8.4  Sensor Failure Handling

- Critical sensor failure (front ultrasonic): prohibit forward motion

- Vision pipeline failure: drop to sensor-only degraded mode, reduce speed

- IMU failure: disable heading-based corrections, continue with US + ToF only

- GPS failure: indoor operation continues; outdoor waypoint navigation pauses

- ToF failure: fall back to ultrasonic for close range

- IR sensor failure: log fault, exclude from safety logic, continue

## 8.5  Low Battery Response

- Stage 1 (battery at 20%): reduce maximum speed to 50%, notify operator

- Stage 2 (battery at 10%): reduce speed to 20%, command vehicle to return to start or stop in place

- Stage 3 (battery at 5%): immediate stop, E-STOP flag, all boards enter low-power wait state

## 8.6  Manual Override Priority

Manual override is always respected in AUX-9 as a safety mechanism. When the operator sends a MANUAL mode command via the web UI, the Raspberry Pi's autonomous commands are suspended immediately. The STM32 prioritizes ESP32 command packets over Pi command packets during MANUAL mode. However, hardware safety checks (obstacle threshold, watchdog) remain active and cannot be overridden even in manual mode.

# 9.  Communication Architecture

## 9.1  Inter-Board Communication Overview

┌─────────────────────────────────────────────────────────────────────┐

│                   INTER-BOARD COMMUNICATION                         │

└─────────────────────────────────────────────────────────────────────┘

┌─────────────┐   UART (3.3V TTL, 115200 baud)   ┌──────────────┐

│ Raspberry   │◄──────────────────────────────────►│  STM32F411  │

│ Pi          │   Commands (Pi→STM32)              │             │

│             │   Telemetry / ACK (STM32→Pi)      │             │

└─────────────┘                                    └──────┬───────┘

│

UART (3.3V TTL, 115200 baud)

│

┌──────▼───────┐

│   ESP32      │

│              │

│  WiFi ◄────► Operator Browser

└──────────────┘

Note: Raspberry Pi and ESP32 do NOT communicate directly.

All coordination passes through STM32F411.

## 9.2  Raspberry Pi ↔ STM32 Link

- Physical: UART (TX/RX, 3.3V TTL logic level)

- Baud rate: 115200 bps

- Direction: bidirectional (Pi commands out, STM32 telemetry in)

- Why UART: Simple, reliable, low-overhead, native on both boards. No additional hardware required. I2C was rejected due to master/slave topology limitations at this data rate. SPI was rejected as unnecessary overhead for this message size.

### Messages from Pi to STM32

- Motion command: [START][MOVE][SPEED_L][SPEED_R][DIRECTION][CHECKSUM][END]

- Mode command: [START][MODE][NEW_MODE][CHECKSUM][END]

- Heartbeat: periodic ping to reset STM32 watchdog

### Messages from STM32 to Pi

- Telemetry: [START][TELEM][US_F][US_R][US_L][US_R][IR_L][IR_R][MODE][BATT][CHECKSUM][END]

- Acknowledgment: [START][ACK][CMD_ID][STATUS][CHECKSUM][END]

- Fault notification: [START][FAULT][FAULT_CODE][CHECKSUM][END]

## 9.3  ESP32 ↔ STM32 Link

- Physical: UART (TX/RX, 3.3V TTL logic level)

- Baud rate: 115200 bps

- Direction: bidirectional

- Why UART: Same rationale as Pi-STM32 link. Consistent protocol across all inter-board links simplifies STM32 software design.

### Messages from ESP32 to STM32

- Manual drive command: [START][MANUAL][SPEED][DIRECTION][CHECKSUM][END]

- Mode switch command: [START][MODE][AUTO/MANUAL/ESTOP][CHECKSUM][END]

- E-STOP button press: [START][ESTOP][CHECKSUM][END]

### Messages from STM32 to ESP32

- Telemetry packet: same format as Pi telemetry (10 Hz)

- Mode confirmation: acknowledgment of mode switch

- Fault alert: forwarded to operator dashboard

## 9.4  Packet Framing Concept

All UART messages in AUX-9 use a simple fixed-format framing scheme:

- START byte: 0xAA (marks beginning of packet)

- Message type byte: identifies the command or data category

- Payload bytes: variable content per message type

- Checksum byte: XOR of all payload bytes for error detection

- END byte: 0x55 (marks end of packet)

The STM32 command parser implements a state machine to receive packets byte-by-byte, validate structure, and dispatch to the appropriate handler. Malformed packets are discarded and a NACK is returned.

# 10.  Power Architecture

## 10.1  Power Domains

┌─────────────────────────────────────────────────────────────────────┐

│                        POWER ARCHITECTURE                           │

└─────────────────────────────────────────────────────────────────────┘

┌────────────────────────────────────────────────────────────────┐

│  Battery Pack  (7.4V 2S LiPo or 7.2V NiMH, 2000–3000 mAh)    │

└───────────────────────┬────────────────────────────────────────┘

│

┌───────────────┼─────────────────────┐

│               │                     │

▼               ▼                     ▼

┌──────────────┐  ┌──────────────┐  ┌───────────────────┐

│ Buck Conv.   │  │ LM7805 / LDO │  │  L298N Motor      │

│ 7.4V → 5V   │  │  5V → 3.3V  │  │  Driver           │

│ (2A+)        │  │  (for GPIO  │  │  (raw battery in) │

└──────┬───────┘  │   peripherals│  └─────────┬─────────┘

│          └──────┬───────┘            │

│                 │              ┌──────┴──────┐

┌──────▼────────────┐    ▼              ▼             ▼

│  Raspberry Pi     │  STM32 3.3V   [Motor A]   [Motor B]

│  ESP32 (5V→3.3V  │  rail

│  internal reg.)   │

│  Sensors (5V)     │

└───────────────────┘

KEY: Motor power and logic power share common ground (GND tie point)

with bulk capacitors (100–470uF) across motor supply rails

## 10.2  Power Rails

- 7.4V (raw battery): Motor supply to L298N only

- 5V regulated: Raspberry Pi, ESP32 (via onboard 3.3V LDO), HC-SR04, VL53L0X, MPU6050, IR sensors

- 3.3V regulated: STM32F411 core and GPIO, logic-level signals

## 10.3  Major Power Risks and Mitigations

- L298N generates large current spikes when switching. Mitigation: 100nF ceramic capacitors across motor terminals, 470uF bulk electrolytic capacitor on 7.4V motor rail, physical separation of motor wiring from signal wiring.Motor switching noise:

- During acceleration, motor current draw can cause transient voltage drops on the shared supply. Mitigation: dedicated buck converter for logic power with at least 2A capacity; bulk capacitors on logic 5V rail.Voltage drop under load:

- Pi requires stable 5V at up to 3A. A voltage drop will cause SD card corruption or random reboot. Mitigation: high-quality buck converter with fast transient response; pi-specific power rail separate from sensor rail.Raspberry Pi brown-out:

- Mitigation: 100nF decoupling capacitors on VDD pins; hardware watchdog will cleanly restart STM32 if reset occurs.STM32 reset from noise:

- LiPo cells damaged below 3.0V/cell (6.0V for 2S). Mitigation: STM32 ADC monitors battery voltage; software low-battery stages prevent full discharge.Battery over-discharge:

## 10.4  Power Isolation Strategy

Motor power and logic power must share a common ground for signal integrity, but the supply rails must be fully isolated by the regulation stage. Key rules:

- Never power the Raspberry Pi or STM32 directly from the battery without regulation

- Place common GND tie point close to battery negative terminal, not near motor driver

- Use separate PCB traces or wiring for motor current paths and signal paths

- Add ferrite beads on 5V logic rail if noise persists after capacitor filtering

# 11.  Text-Based Block Diagrams

## 11.1  Overall System Architecture

┌─────────────────────────────────────────────────────────────────────────┐

│                         AUX-9 SYSTEM OVERVIEW                           │

└─────────────────────────────────────────────────────────────────────────┘

┌─────────────────────────────────────────────────┐

│              RASPBERRY PI 4                     │

│  ┌──────────┐  ┌─────────────┐  ┌───────────┐  │

│  │ Camera   │  │Sensor Fusion│  │ Decision  │  │

│  │ Pipeline │  │   Engine    │  │  Engine   │  │

│  └──────────┘  └─────────────┘  └───────────┘  │

│  ┌─────────────────────────────────────────┐    │

│  │         Mission Manager                 │    │

│  └─────────────────────────────────────────┘    │

└───────────────────────┬─────────────────────────┘

│ UART

┌───────────────────────▼─────────────────────────┐

│              STM32F411                           │

│  ┌────────────┐  ┌───────────┐  ┌────────────┐  │

│  │ Command    │  │  Motor    │  │  Safety    │  │

│  │ Parser     │  │  Control  │  │  Monitor   │  │

│  └────────────┘  └───────────┘  └────────────┘  │

│  ┌──────────────────────────────────────────┐    │

│  │  US Manager + IR Monitor + Watchdog      │    │

│  └──────────────────────────────────────────┘    │

└─────────────────────────────────────────────────┘

│ UART               │ PWM+DIR        │ UART

▼                    ▼                ▼

┌──────────┐        ┌──────────┐    ┌──────────────┐

│  ESP32   │        │  L298N   │    │  Telemetry   │

│ WiFi/Web │        │ H-Bridge │    │  to ESP32    │

└──────────┘        └────┬─────┘    └──────────────┘

│ WiFi          ┌────┴────┐

▼              ▼          ▼

[Browser]      [Motor A]  [Motor B]

## 11.2  Sensor-to-Board Connections

┌─────────────────────────────────────────────────────────────────────┐

│                  SENSOR-TO-BOARD CONNECTIONS                        │

└─────────────────────────────────────────────────────────────────────┘

RASPBERRY PI                           STM32F411

─────────────                          ───────────

│ [Pi Camera]   ◄─── CSI ribbon        │ [HC-SR04 Front] ◄─ GPIO/Timer

│ [MPU6050]     ◄─── I2C (0x68)        │ [HC-SR04 Rear]  ◄─ GPIO/Timer

│ [VL53L0X]     ◄─── I2C (0x29)        │ [HC-SR04 Left]  ◄─ GPIO/Timer

│ [NEO-6M GPS]  ◄─── UART (/dev/ttyS0) │ [HC-SR04 Right] ◄─ GPIO/Timer

│ [IR Sensor L]   ◄─ GPIO

│ [IR Sensor R]   ◄─ GPIO

│ [Battery ADC]   ◄─ ADC pin

## 11.3  Control Loop Flow

┌─────────────────────────────────────────────────────────────────────┐

│                    CONTROL LOOP FLOW                                │

└─────────────────────────────────────────────────────────────────────┘

Sensors (all) ──▶ Raspberry Pi Perception ──▶ Sensor Fusion

│

▼

Decision Engine

(obstacle avoidance,

speed/direction cmd)

│

UART Command

│

▼

HC-SR04/IR ──▶ STM32 Safety Monitor ──▶  Motor Control Task

(local check)   (E-STOP if needed)         │

▼

L298N ──▶ Motors

Feedback path:

STM32 sensor data ──▶ UART ──▶ Raspberry Pi (closes perception loop)

# 12.  Task and Software Architecture

| Board | Software Module | Responsibility |
| --- | --- | --- |
| Raspberry Pi | Camera Pipeline | Frame capture, resize, object detection via OpenCV / TFLite, annotated output |
|  | IMU/GPS Reader | Read MPU6050 via I2C, parse NMEA from GPS UART, timestamp all samples |
|  | Sensor Fusion Engine | Merge camera, IMU, GPS, ToF into unified environment model; detect stale/missing data |
|  | Decision Engine | Obstacle avoidance logic, path planning, speed/direction command generation |
|  | Mission Manager | Load/store missions, track goal state, manage mode transitions, coordinate with STM32 |
|  | UART Comm Module | Frame and send commands to STM32; receive telemetry/ack; handle timeout/retry |
| STM32F411 | Command Parser | Decode incoming UART packets from Pi and ESP32; validate checksum; queue to actuator |
|  | Motor Control Task | Generate PWM signals for L298N; implement acceleration ramping; direction logic |
|  | Ultrasonic Manager | Trigger HC-SR04 sequence, measure echo pulses via timer capture, compute distances |
|  | IR Monitor | Poll IR sensors at fixed interval; detect obstacles or edge; trigger safety response |
|  | Safety Monitor | Watchdog timer; check comm timeout; enforce minimum safe distances; assert E-STOP |
|  | Telemetry Reporter | Send encoder counts, sensor readings, mode, battery level to ESP32 at 10 Hz |
| ESP32 | WiFi Access Point | Host soft AP (192.168.4.x), manage connected clients, SSID/password config |
|  | Async Web Server | Serve control dashboard HTML/JS; handle HTTP API endpoints for commands |
|  | Command Interface | Parse joystick/button inputs from web UI, translate to UART command packets for STM32 |
|  | Telemetry Sender | Receive telemetry from STM32 via UART; push to web dashboard via WebSocket or polling |
|  | Mode Switch Handler | Accept AUTO/MANUAL/E-STOP mode requests from user; relay to STM32; notify Pi via STM32 |

## 12.1  Software Module Interactions

### Raspberry Pi Internal Interactions

- Camera Pipeline feeds detected object data to Sensor Fusion Engine

- IMU/GPS Reader feeds pose data to Sensor Fusion Engine

- Sensor Fusion Engine outputs unified world model to Decision Engine

- Decision Engine outputs motion commands to UART Comm Module

- Mission Manager supervises Decision Engine and triggers mode changes

- UART Comm Module receives telemetry from STM32 and delivers to Mission Manager and Sensor Fusion

### STM32 Internal Interactions

- Command Parser receives UART packets and dispatches to Motor Control Task or Mode Handler

- Safety Monitor runs independently in a high-priority interrupt context and can override Motor Control Task output

- Ultrasonic Manager populates a shared distance array consumed by Safety Monitor

- IR Monitor populates binary obstacle flags consumed by Safety Monitor

- Telemetry Reporter reads from all sensor and mode state variables and transmits to ESP32

- Watchdog / Fault Handler resets STM32 on lockup and asserts E-STOP on any unhandled exception

### ESP32 Internal Interactions

- Async Web Server receives HTTP requests and routes to Command Interface

- Command Interface translates web inputs to UART packets and sends to STM32

- Telemetry Sender receives UART packets from STM32 and pushes to connected browser via WebSocket

- Mode Switch Handler intercepts mode-change requests from the UI and formats them as priority UART packets

# 13.  Assumptions and Design Decisions

## 13.1  Drive Configuration

- 2-wheel differential drive (2WD): Two driven rear wheels, one or two passive front casters. This is chosen for simplicity, low cost, and ease of steering control in software.

- No servo steering: Direction is achieved purely by differential motor speed. This eliminates the need for a servo driver and associated complexity.

## 13.2  Operating Environment

- Indoor primary, outdoor secondary: Phase 1 is designed primarily for flat indoor surfaces (tile, hardwood, concrete). GPS is optional in Phase 1.

- Flat surface assumption: No significant slope handling in Phase 1. Slopes cause uneven motor loading that would require encoders to compensate — deferred to Phase 3.

## 13.3  Speed and Size

- Maximum speed: 0.5 m/s (approximately 1.8 km/h). Low speed chosen for safety, reliable sensor response time, and to stay within the effective reaction distance of ultrasonic sensors at 20 Hz polling.

- Chassis size: Approximately 25 cm x 20 cm (L x W). Consistent with standard DIY robot chassis available within budget.

## 13.4  Software Stack

- Raspberry Pi: Python 3, OpenCV, smbus2 (I2C), pyserial, pynmea2 for GPS, optional TFLite for AI inference

- STM32F411: Bare-metal C using STM32 HAL library, timer-based PWM, UART interrupt receive, hardware watchdog (IWDG)

- ESP32: Arduino framework with ESPAsyncWebServer library, ArduinoJson for packet parsing

## 13.5  Communication Assumptions

- UART baud rate: 115200 bps on all inter-board links — sufficient for command rate and telemetry without hardware flow control

- No CAN bus, no ROS, no Ethernet in Phase 1 — all are cost and complexity overkill for this stage

- WiFi range: Operator assumed to be within 20–30 meters. No long-range operation in Phase 1.

# 14.  Limitations of Phase 1 Architecture

## 14.1  What the System Can Do Well

- Real-time obstacle detection and avoidance using ultrasonic sensors

- Stable manual control via WiFi web interface

- Basic autonomous navigation using reactive obstacle avoidance

- Reliable safety shutdown via hardware watchdog

- Live telemetry display to operator

- Modular enough to upgrade individual boards without redesigning the whole system

## 14.2  What the System Cannot Do Yet

- Precise dead-reckoning or odometry: No wheel encoders in Phase 1 means the vehicle cannot accurately track how far it has travelled.

- Advanced path planning: Phase 1 uses reactive avoidance, not pre-planned paths or SLAM. The vehicle reacts to obstacles but does not build a map.

- High-speed navigation: Maximum safe speed is limited by sensor polling rate and absence of encoders.

- GPS waypoint navigation: GPS hardware is present but full waypoint navigation logic is Phase 2 scope.

- Slope or rough terrain handling: Only flat surfaces are reliable in Phase 1.

- Multi-sensor redundancy on all axes: Front obstacle detection has redundancy (US + ToF + IR), but lateral and rear coverage relies on ultrasonic only.

## 14.3  What Will Be Improved in Later Phases

- Phase 2: Add wheel encoders for odometry, implement PID speed control, integrate GPS waypoint navigation, refine sensor fusion with Kalman filter

- Phase 3: Implement ROS 2 on Raspberry Pi, SLAM for indoor mapping, upgrade to servo-based steering for more precise control

- Phase 4: Advanced AI models for scene understanding, multi-vehicle coordination, cloud telemetry logging

# 15.  Phase 1 Deliverable Summary

## 15.1  Final Architecture Summary

AUX-9 Phase 1 establishes a complete, professional-grade system architecture for a low-cost autonomous hybrid vehicle. The design separates intelligence (Raspberry Pi), real-time control (STM32F411), and communication (ESP32) into dedicated boards, connected via UART. A six-subsystem model — Perception, Decision, Control, Communication, Safety, and Power — provides clear boundaries for development, testing, and scaling.

The architecture is realistic, buildable within the INR 7,000–8,000 hardware budget (excluding the three core boards), and follows industry-standard embedded systems design principles including hardware safety authority, fail-safe defaults, watchdog supervision, and fault-tolerant degraded operation.

## 15.2  Subsystem Summary

| Subsystem | Board | HW Components | Phase 1 Scope |
| --- | --- | --- | --- |
| Perception | Raspberry Pi | Camera, MPU6050, VL53L0X, GPS, US, IR | Capture, preprocess, relay to fusion |
| Decision | Raspberry Pi | Software only | Obstacle avoidance, basic path planning |
| Control | STM32F411 | L298N, DC motors | PWM control, direction, ramping |
| Communication | ESP32 | WiFi module (built-in) | Web UI, manual relay, telemetry |
| Safety | STM32 primary | Watchdog timer, IR, US | E-STOP, watchdog, comm timeout handling |
| Power | All (passive) | LiPo, LM7805, buck converters | Dual-rail power, noise isolation |

## 15.3  Board Responsibility Summary

| Board | Primary Role | Real-time Class | Key Responsibilities |
| --- | --- | --- | --- |
| Raspberry Pi 4 | AI & Decision Engine | Non / Soft real-time | Camera pipeline, sensor fusion, path planning, mission manager, mode arbiter |
| STM32F411 | Real-time Control & Safety | Hard real-time | PWM motor control, safety watchdog, command parsing, emergency stop authority |
| ESP32 | Communication & Manual I/O | Soft real-time | WiFi access point, web dashboard, manual command relay, telemetry display |

## 15.4  Sensor Summary

| Sensor | Board | Data Provided | Read Rate | Subsystem | Failure Mode | Failure Response |
| --- | --- | --- | --- | --- | --- | --- |
| HC-SR04 (x4) | STM32 | Distance 2–400 cm | 20 Hz | Perception / Safety | No pulse / frozen value | Mark zone as 'unknown-danger', halt if front sensor fails |
| MPU6050 | Raspberry Pi | Accel + Gyro (6-DOF) | 50–100 Hz | Perception / Fusion | I2C timeout / garbage data | Disable IMU fusion channel, degrade to dead-reckoning or halt |
| Pi Camera | Raspberry Pi | RGB frames (480p+) | 15–30 fps | Perception (vision) | No frames / crash | Drop to sensor-only nav or enter degraded mode |
| VL53L0X | Raspberry Pi | Precise range 0–2 m | 30–50 Hz | Perception / Safety | I2C timeout | Rely on ultrasonic for short-range; log fault |
| NEO-6M GPS | Raspberry Pi | Lat/Lon/Alt, NMEA | 1–5 Hz | Decision (position) | No fix / weak signal | Indoor: GPS optional. Outdoor: enter wait-fix state before moving |
| IR Sensors (x2) | STM32 | Binary obstacle / edge detect | 20 Hz | Safety / Backup | Always-high or always-low | Flag sensor faulty; secondary backup so degrade gracefully |

## 15.5  Recommended Next Phase

Upon completion and validation of the Phase 1 architecture review, the recommended next phase is:

Phase 2: Detailed Hardware Design and Bill of Materials

Phase 2 scope: Select final component models with exact part numbers and prices, produce a verified BOM within budget, create circuit schematics for all inter-board connections and power regulation, define PCB/stripboard layout strategy, and prototype the power subsystem for noise validation before full assembly.

END OF PHASE 1 — SYSTEM ARCHITECTURE DESIGN DOCUMENT

AUX-9 Autonomous Hybrid Intelligence Vehicle System